Project Synopsis

1. Project Title

Visual Microphone Pro: Real-Time Sub-Pixel Acoustic Reconstruction from Video Streams using GPU-Accelerated Quaternionic Riesz Pyramids and Neural Speech Enhancement

2. Abstract / Summary

This project presents an advanced Python-based computer vision system that recovers audible acoustic signals from high-speed and standard consumer video sequences without physical microphones. Sound waves hitting physical objects (such as chip bags, plant leaves, or glass) cause high-frequency, sub-pixel surface vibrations. By extending MIT’s original Visual Microphone concept, this project develops a GPU-accelerated pipeline utilizing Quaternionic Riesz Pyramids for phase-based temporal motion analysis.

To overcome the fundamental limitations of prior hand-crafted MATLAB implementations—namely high computational latency, vulnerability to camera shake, strict high-FPS requirements, and sensor noise—this project introduces:

- Camera Motion Decoupling via Enhanced Correlation Coefficient (ECC) image registration.

- CMOS Rolling Shutter Temporal Super-Resolution to recover acoustic frequencies exceeding the Nyquist limit of consumer 30/60 FPS cameras.

- Deep Learning-Based Speech Enhancement to eliminate camera read noise and environmental interference.

- An End-to-End Production-Grade Interactive Web/Desktop Application providing real-time audio extraction, region-of-interest (ROI) tracking, and STFT spectral visualization.

3. Problem Statement & Motivation

- Limitations of Existing Work: Traditional acoustic recovery from video relies on MATLAB-based steerable pyramids that suffer from heavy computational overhead, making real-time execution impossible.

- Sensitivity to Macro-Motion: Small camera shakes or rigid-body object movements completely corrupt sub-pixel phase extraction.

- Hardware Constraints: Standard algorithms require expensive high-speed cameras (≥2,000 FPS) due to the Nyquist sampling theorem (fsound<FPS2), making them impractical for standard smartphone cameras.

- Noise Contamination: Raw visual phase extraction inherently yields low Signal-to-Noise Ratios (SNR) due to camera sensor read noise and Poisson photon noise.

4. Research Objectives

- Phase 1 Objectives (Months 1–3):

  - Build a high-performance Python core engine matching and exceeding the 2014 MIT CSAIL baseline.

  - Implement PyTorch/CUDA GPU acceleration for real-time pyramid decomposition.

  - Introduce Rolling Shutter Analysis and Motion Decoupling algorithms to extract audio from standard 30/60 FPS video under slight camera movement.

  - Perform quantitative benchmarking using PESQ, STOI, and SNR metrics against ground-truth microphone recordings.

  - Draft and submit a double-column IEEE/ACM conference research paper.

- Phase 2 Objectives (Months 4–9):

  - Integrate pre-trained deep neural speech enhancement models (DeepFilterNet / DCCRN) to clean extracted raw audio.

  - Implement Automatic Region of Interest (Auto-ROI) Detection using spatial phase variance maps.

  - Develop a full-featured, cross-platform interactive GUI (PyQt6 / Streamlit) featuring real-time audio playback, STFT spectrogram visualizers, and FFmpeg video renderers.

  - Publish an open-source PyPI library (pip install visual-mic) and submit an expanded manuscript to an open-access peer-reviewed journal.

5. Proposed Methodology & System Architecture

- Pre-Processing & Video Registration: Frames are stabilized using Enhanced Correlation Coefficient (ECC) alignment to decouple macro camera movements from microscopic vibrations.

- Spatial-Temporal Phase Decomposition: Each frame is decomposed into a multi-scale Laplacian pyramid. A 2D Riesz transform (central difference operators) extracts local quaternionic phase and orientation.

- Temporal Phase Filtering & Unwrapping: Local frame-to-frame phase differences are unwrapped and temporally filtered through an IIR bandpass filter tuned to target speech/acoustic frequencies.

- Rolling Shutter Super-Resolution: For consumer cameras, row-by-row CMOS readout timing is exploited to temporally upsample pixel rows into microsecond acoustic samples (Freadout=Frame Height×FPS).

- Phase Fusion & AI Post-Processing: Multi-scale phase signals are fused using local SNR weighting. The composite 1D signal is passed through a deep speech enhancement neural network to output clean, intelligible audio (.wav).

6. Expected Novelties & Scientific Contributions

- GPU Acceleration: 10×-50× faster execution than conventional MATLAB pyramid toolboxes.

- Sub-Nyquist Recovery: Successful audio extraction (>1 kHz) from regular 30/60 FPS smartphone sensors using rolling shutter geometry.

- Robustness to Shake: Elimination of false phase artifacts caused by handheld camera drift.

- End-to-End Product: A open-source Python tool and web/desktop application combining vision, physics, and signal processing.

7. Tools, Libraries & Hardware Requirements

- Programming Language: Python 3.10+

- Core Computational Stack: PyTorch (CUDA acceleration), NumPy, SciPy, OpenCV

- Audio & Signal Processing: librosa, pPESQ, pystoi, scipy.signal, DeepFilterNet

- Application & UI Framework: Streamlit / PyQt6, FFmpeg

- Hardware Needed: Standard PC/Laptop equipped with an NVIDIA GPU (GTX 1060 / RTX 2060 or higher recommended).