1. Fundamental Mathematics & Physics Improvements

- Phase Unwrapping: Standard phase extraction uses I=atan2y,x or arccos, which wraps phase values between -π and π. Implement robust 2D/3D temporal phase unwrapping (e.g., Goldstein’s 2D algorithm) to handle large-amplitude vibrations without audio distortion.

- Higher-Order Riesz Transform Kernels: Replace 3x3 central-difference spatial gradient kernels (-0.5,0,0.5) with higher-order, frequency-tuned Riesz filters or monogenic signals (Hilbert-Riesz filters) to capture fine spatial orientations more accurately.

- Sub-Pixel Motion Estimation: Integrate phase-based optical flow or local spatial phase gradients to improve spatial sensitivity to sub-pixel displacements (<0.01 px).

2. Audio Quality & Signal Processing Improvements

- Adaptive Denoising & Spectral Subtraction: Standard bandpass filtering leaves substantial camera sensor noise (e.g., Poisson/Gaussian read noise). Implement Wiener filtering, spectral subtraction, or deep learning-based audio speech enhancement (e.g., DeepFilterNet) on the raw output waveform.

- Multi-Scale / Multi-Orientation Phase Fusion:

  - Instead of averaging phase signals equally across pyramid levels, implement Weighted Signal Alignment.

  - Use local spatial coherence or local SNR weighting so high-SNR regions (e.g., crisp edges on a vibrating object) contribute more than noisy, flat-textured regions.

- Audio Post-Processing Pipeline: Automatically scale, normalize, and match the sampling rate (Fs) to standard audio outputs (e.g., 16 kHz or 44.1 kHz) using high-quality anti-aliasing resampling (scipy.signal.resample_poly).

3. Handling Real-World Video Issues (Robustness)

- Rigid-Body & Macro Motion Decoupling:

  - The Problem: Global camera shake or human movement swamps the micro-vibrations.

  - The Fix: Implement pre-processing image registration using ECC (Enhanced Correlation Coefficient) or feature-based optical flow (KLT tracker) to cancel global camera drift before phase temporal filtering.

- Rolling Shutter Exploitation:

  - The Problem: Consumer videos at 30 FPS can only capture sound frequencies up to 15 Hz (Nyquist limit).

  - The Fix: Exploit CMOS rolling shutter readout timing. Because each row is exposed sequentially, reconstruct 1D temporal signals at thousands of Hertz (Freadout=height×FPS).

- Automatic ROI (Region of Interest) Detection: Automatically highlight and extract audio from high-vibration regions using edge strength and temporal phase variance maps, rather than processing the entire frame.

4. Performance, Optimization & Hardware Acceleration

- GPU Acceleration:

  - Convert OpenCV/SciPy matrix convolutions and FFT/IIR filters to PyTorch, CuPy, or CUDA.

  - FFTs and pyramid operations scale massively on GPU, increasing speed by 10× to 50×.

- Batch & Real-Time Stream Processing:

  - Refactor processing into a multi-threaded producer-consumer pipeline (e.g., using queue.Queue or multiprocessing).

  - Read frames, process pyramids, and stream audio in chunked buffers to enable real-time execution.

- Memory Management: Reduce memory allocation during video processing by reusing static NumPy memory buffers instead of allocating new pyramid arrays for every frame.

5. Application & User Interface (GUI) Improvements

- Interactive Web/Desktop UI: Build a UI using Streamlit, Gradio, or PyQt6 featuring:

  - Interactive video upload & ROI bounding-box selection tool.

  - Real-time audio waveform and spectrogram display (matplotlib or librosa).

  - Live frequency-tuning sliders (Bandpass low/high cutoffs, amplification factor α).

- Export Options: Provide side-by-side video rendering with synchronized reconstructed audio (ffmpeg-python integration) and multiple audio export formats (.wav, .mp3, .flac).

6. Research & Publishing Roadmap

- Quantitative Evaluation Metrics: Benchmark reconstructed audio against ground-truth microphone recordings using:

  - SNR (Signal-to-Noise Ratio)

  - STOI (Short-Time Objective Intelligibility)

  - PESQ (Perceptual Evaluation of Speech Quality)

- Code Packaging & Licensing:

  - Package the Python library neatly (pip install visual-mic).

  - Write clean Sphinx/MkDocs documentation with unit tests (pytest).

  - Open-source the repository with clear MIT or BSD licenses and citation notices.